|  | LO: To use sequence, selection and repetition Context: Countdown clock timer Success criteria: - All children will create a visual 30 second timer - Mastery: children will control their timer using a push-to-make switch - Greater depth: children will add in buzzer beeps, which increase in frequency near the end of the timer |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |
| Prior Learning: Prior to this lesson, it would be useful if children have had experience using inputs to control outputs and degrees in maths. |  |  | Resources and Safety: Along with Crumbles and computers, you will need: a battery box (NO rechargeable batteries), a servo with crumbliser, a buzzer, a push-to-make switch and nine croc-leads. You will also need craft materials and optionally the printed clock template. |  | Key Vocabulary: The following terminology may be used: software, programming, input, output, sequence, algorithm, loop, repetition, servo, degrees. |  |

|  Teacher Input: | Introduce the lesson by showing children a video of the Countdown clock in action. Ask the children what they think it’s for? How long is it? How many degrees does the second hand turn? |
| --- | --- |
|  Discussion: | Explain to the class that you are going to be making your own countdown clocks, using a servo to move the hand. Explain that servos have a range of movement which is 180 degrees and in the Crumble software we use the servo block, with a number between -90 and 90, to tell the servo where to move. Get children to discuss the following: In 30 seconds, the hand needs to move 180 degrees – how many degrees do we need to move each second? How could we do this with the Crumble? |
|  Activity: | Children work on creating a countdown clock – encourage the use of a loop to achieve this and not a large list of instructions. |
|  Teacher Input: | Explain to the class that it would be useful to be able to start the timer at the push of a button, and to be able to reset it afterwards without having to use the battery switch. Can children add in a push-to-make switch? |
|  Activity | Children add a push-to-make switch into their project, which starts the countdown timer and then resets the clock after it has finished. Greater Depth – can children add in a buzzer to their project? Can they make it beep quicker when the timer is nearly finished? |

|  Teacher Input: | Explain the process of making the clock to put everything into. |
| --- | --- |
|  Activity | Children make their clocks either using the template provided or using a compass and a protractor to draw the correct angles. |
|  Plenary: | Give children the chance to look at one another’s projects. Are they different? Why? How could you have improved your project? How could you use what you’ve learnt today in a project in the future? |
|  Next steps: | - Draw an accurate clock face, with the correct angles (maths); - Use the timers within a game; - Design and create their own clock for the project to go into (D&T); - Write descriptions in 30 seconds; - Write a story which lasts for 30 seconds. |